VIREON

Candidate Assessment | 1-Week Project

# Background

Vireon deploys IoT-based monitoring systems at industrial facilities. Our edge devices collect electrical parameters from energy meters every 5 minutes and transmit data for analysis.

You have been provided a sample data log from a live deployment with two monitoring units (Shed_01 and Shed_02). Use this data to complete the 2 assigned tasks – 1 each for assessing the 2 core competencies for this role:

## Data Overview

Key columns in the dataset:

- Timestamp, Date, Time, Hour — When the reading was taken

- Device_ID, Location — Which meter/shed the reading is from

- ToD_Period, Rate_Rs_kWh — Time-of-Day tariff (PEAK/OFFPEAK) and rate

- Watts_Total, kW_Total, kVA_Total — Power consumption

- V_R, V_Y, V_B, VLL_Avg, VLN_Avg — Voltage readings (3-phase)

- Current_R, Current_Y, Current_B, Current_Total — Current readings

- PF_Avg, PF_R, PF_Y, PF_B — Power factor readings

- Energy_kWh, Daily_Energy_Wh — Cumulative and daily energy

- Fire_Risk_Level — Derived safety indicator (NORMAL/HIGH/WARNING)

- Voltage_Unbalance_Pct, Current_Unbalance_Pct — Power quality metrics

- Pi_Temp_C, Pi_CPU_Pct, Pi_Mem_Pct — Edge device health

# Field IoT Engineer — Problem Statement

## Feature 1: Data Continuity Checker

Problem: In field deployments, data gaps occur due to connectivity issues, device reboots, or meter communication failures. We need to detect and report these gaps.

Task:

- Write a Python script that analyzes timestamps in the data log

- Expected reading interval: ~5 minutes. Flag gaps > 10 minutes as potential issues

- For each device (Shed_01, Shed_02), report: total readings, missing intervals, data completeness %

- Identify if gaps are device-specific or system-wide (affecting both sheds)

- Output: Summary report with gap timestamps and duration

Evaluation Criteria:

- Correct time-series handling

- Accurate gap detection

- Per-device analysis

- Practical insights from the analysis

# Data & Software Engineer — Problem Statement

## Feature 2: Power Quality Alert System

Problem: Poor power quality (voltage unbalance, low power factor, neutral current) can damage equipment and increase costs. We need to flag anomalies.

Alert Thresholds:

- Voltage Unbalance > 2% — Warning; > 5% — Critical

- Power Factor < 0.9 — Warning; < 0.85 — Critical (attracts penalty)

- Neutral Current > 10% of phase current average — Warning

- Fire_Risk_Level = 'WARNING' or 'HIGH' — Flag immediately

Task:

- Write a Python script that scans the data log for power quality issues

- Generate an alert log with: timestamp, device, alert type, severity, actual value

- Summarize: total alerts by type, alerts by device, most frequent issues

- Bonus: Suggest possible causes for observed patterns

Evaluation Criteria:

- Correct threshold implementation

- Comprehensive alert capture

- Useful summary and insights

- Code quality and documentation

# Submission Guidelines

Deadline: 7 days from receiving this assignment

Deliverables:

- Python scripts (.py files) — one per feature

- README file explaining: how to run the code, any dependencies, your approach

- Sample output from your scripts

- Brief notes on: challenges faced, assumptions made, ideas for improvement

Submit via: Email to saikat@vireontech.in or GitHub repo link

Questions? Reach out if anything is unclear. Asking good questions is part of the evaluation.

## What We're Looking For

- Working code that solves the problem (doesn't need to be perfect)

- Clear thinking and problem breakdown

- Sensible handling of edge cases and data issues

- Readable, maintainable code with comments

- Honest documentation of what works and what doesn't

Good luck! We're excited to see your approach.